BALANCING CODE NO. 4

(BC4)

TERRE  PROCESSES

CONTENTS

(This contents page does not form part of the Grid Code)

Paragraph No/Title	Page Number

BC4.1   INTRODUCTION	2

BC4.2   OBJECTIVE	2

BC4.3   SCOPE	2

BC4.4  REQUIREMENTS FOR BM PARTICIPANTS WHO WISH TO PARTICIPATE IN TERRE	2

BC4.4.1   RR Provider Prequalification 	2

BC4.4.2   Minimum Technical Requirements	3

BC4.4.3   Prequalification Timelines	3

BC4.4.4    Requalification Criteria……………………………………………………….……..………….3

BC4.5  SUBMISSION of TERRE RELATED DATA BY BM PARTICIPANTS	3

BC4.5.1   Communication from BM Participants to The Company	3

BC4.5.2   RR Provider Data submission before TERRE Gate Closure	4

BC4.5.3   Re-Submission of parameters by BM Participants before TERRE Gate Closure	4

BC4.5.4   Defaulting rules for TERRE bids	4

BC4.6   Processing of TERRE Bids before passing to the TERRE Central Platform	5

BC4.6.1   Cases where a TERRE Bid will be Restricted	5

BC4.7   Instructing BM Participants	5

BC4.7.1   Communication from The Company to BM Participants	5

BC4.7.2   Creating RR Instructions from RR Acceptances 	5

BC4.7.3   Cases where RR Instructions may not be issued	5

BC4.7.4   Infeasability of RR Acceptances	6

BC4.8   Publication of TERRE Data	6

BC4.8.1   Publication of Data at the European level	6

BC4.8.2   Publication of Data at the National Level	6

BC4.9   Outages of computer systems leading to the suspension of the TERRE market	6

BC4.1	INTRODUCTION

Balancing Code No 4 (BC4) sets out the procedures for:

(a)	prequalifation requirements for participation in TERRE by BM Participants;

(b)	submission of data by BM Participants wishing to take part in TERRE;

(c) 	validation of data from BM Participants wishing to take part in TERRE;

(d)	issuing of RR Instructions; and

(e) 	publication of TERRE related data.

BC4.2	OBJECTIVE

This procedure facilitates the participation of BM Participants in the TERRE market. Particpation in TERRE is voluntary for BM Participants.

BC4.3	SCOPE

BC4 applies to :-

- The Company;

- BM Participants;

(b)	Externally Interconnected System Operators; and

(c)	Network Operators.

BC4.4	REQUIREMENTS FOR BM PARTICIPANTS WHO WISH TO PARTICIPATE IN TERRE

The Company shall ensure that each relevant Balancing Service prequalification process shall, as a minimum, require the RR provider to submit a self-certification of the RR Minimum Technical Requirements as defined in BC4.4.1 and BC4.4.2.

BC4.4.1	All BM Participants who wish to participate in TERRE must have successfully completed the prequalification process to be a RR provider as detailed in BC5.

BC4.4.2	All BM Participants who wish to participate in TERRE must have the following 				capabilities:-

- BM Participants must have the ability to submit data and receive instructions by the use of electronic data communication facilities as provided for in CC.6.5.8 or ECC.6.5.8.

- BM Participants must be capable of following an RR Instruction issued by The Company.

- BM Participants must be able to provide Physical Notifications.

- BM Participants must be able to provide a subset of Dynamic Parameters (as detailed in BC4.5.2).

- BM Participants must provide operational metering for their total output and for any individual component that may have an output greater than 1MW. This metering must have the following accuracy;

  - For a BM Unit with either a Generation Capacity greater than 100MW or Demand Capacity greater than 100MW, a metering accuracy better than 0.5%.

  - For a BM Unit with a Generation Capacity greater than 10MW but less than or equal to 100MW or Demand Capacity greater than 10MW but less than or equal to 100MW, a metering accuracy better than 1%.

  - For all other BM Units, an accuracy better than 2.5% is required.

- BM Participants must have the ability to inform The Company if their availability changes using Export and Import Limits.

- For BM Participants connected within a User System, BM Participants must be capable of informing Network Operators of their availability and aciviation in realtime if required.

BC4.4.3	Prequalification Timelines

European Regulation (EU) 2017/1485 gives the following minimum timescales for the prequalification process;-

(a)	Within 8 weeks of a formal application from the BM Participant, The Company shall confirm the application is complete (from the perspective of information provision).

(b)	If the application is incomplete, the BM Participant shall provide the missing evidence within 4 weeks of the a request from The Company or it will be presumed that the application has been withdrawn.

(c)	Within 3 months of confirming that all information has been provided, The Company shall confirm if the potential BM Participant meets the requirements in BC4.4.2. For the avoidance of doubt, The Company will not carry out independent tests but will review the evidence provided.

BC4.4.4	Requalification criteria

Under certain conditions, an BM Participant must requailify.

(a)	Every five years, a BM Participant must requalify to the technical requirements in BC4.4.2 and according to the timescales in BC4.4.3.

(b)	If at any time, a BM Participant becomes aware of changes to the configuration forming the BM Unit, that means the minimum technical requirements in BC4.4.2 can no longer be met, then that BM Participant must withdraw from TERRE and must requailify.

BC4.5	SUBMISSION OF TERRE RELATED DATA BY BM PARTICIPANTS

BC4.5.1	Communication from BM Participants to The Company

(a)	Submission of data specified in BC4.5.2 will be by use of electronic data communications facilities, as provided for in CC.6.5.8 or ECC.6.5.8.

(b)	In the event of a failure of the electronic data communication facilities, the data used in the TERRE auction will be based on the most recent data received and acknowledged by The Company. In the event of missing data, it will be assumed the BM Participant did not wish to submit data for the relevant TERRE Auction Period.

(c)	Planned Maintenance Outages will normally be arranged to take place during periods of low data transfer activity.

(d)	Upon any Planned Maintenance Outage, or following an unplanned outage described in BC4.5.1(b) (where it is termed a "failure") in relation to a pre-TERRE Gate Closure:

(i)	If a BM Participant has submitted Physical Notifications and a 		TERRE Bid for a TERRE Auction Period the BM Particpant 			should continue to act in relation to any period of time in 				accordance with the Physical Notifications current at the time of 		the start of the Planned Maintenance Outage or the computer 			system failure in relation to each such period of time subject to the 		provisions of BC2.5.1. Depending on when in relation to TERRE 			Gate Closure the planned or unplanned maintenance outage arises, 			such operation will either be operation in preparation for the 			relevant output in real time, or will be operation in real time. 			No further submissions of BM Participants data should be 			attempted. Plant failure or similar problems causing 				significant deviation from the Physical Notification should 			be notified to The Company by the submission of a revision 			to Export and Import Limits in relation to the RR Provider 			so affected;

(ii)		No data will be transferred from The Company to the 				Balancing Mechanism Reporting Agent (BMRA) until the 			communication facilities are re-established.

BC4.5.2	RR Provider Data submissions before TERRE Gate Closure

To participate in a TERRE auction, a BM Participant must have prequalified and must submit a TERRE Bid covering at least one of the TERRE Activation Periods within the TERRE Auction Period.

In addition to a valid TERRE Bid, a sub-set of Balancing Mechanism parameters are also required covering the TERRE Auction Period and the Settlement Periods immediately before and after the TERRE Auction Period (to allow ramping before and after).

If a BM Participant is active in the Balancing Mechanism the only additional data needed to participate in a TERRE auction is a valid TERRE Bid covering the relevant times.

For a BM Participant that is not active in the Balancing Mechanism, the following subset of parameters are required with exceptions as noted below:

(a)	Physical Notifications

Physical Notifications follow the same format and rules as covered in BC1 and BC2 with the following exceptions;

- A BM Participant that is not active in the Balancing Mechanism but wishes to participate in TERRE is only required to have submitted Physical Notifications covering the TERRE Auction Period and the Settlement Periods immediately before and after the TERRE Auction Period for which they have submitted a TERRE Bid.

- Defaulting rules as described in the Data Validation, Consistency and Defaulting Rules will only apply to Settlement Periods for which the BM Participant previously submitted Physical Notifications for the previous Operational Day.

(b)	Export and Import Limits

For a BM Participant that is not active in the Balancing Mechanism but wishes to participate in TERRE, these are the same as described in BC1 and BC2.

- Run Up Rate and Run Down Rates

For a BM Participant that is not active in the Balancing Mechanism but wishes to participate in TERRE these are the same as described in BC1 and BC2.

- For a BM Participant that is not active in the Balancing Mechanism but wishes to participate in TERRE, the other Dynamic Parameters listed in BC1.A.1.5 are not required.

TERRE Bids must follow the formats and rules in the TERRE Data Validation and Consistency Rules.

BC4.5.3	Re-submission of parameters by BM Participants before TERRE Gate Closure

The rules outlined in BC1 and BC2 for the revision of Physical Notifications, Export and Import Limits, Run Up Rates and Run Down Rates also apply for TERRE.

TERRE Bids can be revised up to TERRE Gate Closure in order to be used in the TERRE auction (as described in the TERRE Data Validation and Consistency Rules).

BC4.5.4	Defaulting rules for TERRE Bids

TERRE Bids will not be defaulted using previously submitted values. This is due to the ability to link TERRE Bids and the re-use of sequence numbers. Hence a BM Participant wishing to particpitate in a particular TERRE auction must submit RR Bids specifically covering the relevant TERRE Activation Periods.

.

BC4.6			Processing of TERRE Bids before passing to the TERRE Central Platfom

BC4.6.1		Cases where a TERRE Bid will be Restricted

TERRE Bids will be passed to the TERRE Central Platform but will be flagged as Restricted under the following cases:-

- Data within the submission does not conform to formats required as detailed in the TERRE Data Validation and Consistency Rules (e.g. missing or incorrect keywords, data in the wrong order, corrupted files etc).

- If a TERRE Bid does not have a corresponding Physical Notification, the TERRE Bid will be flagged as Restricted.

- If a TERRE Bid will result in violating a System Constraint, it will be flagged as Restricted.

- If a BM Participant has already been instructed for an Ancillary Service or for Reserve, a TERRE Bid may need to be flagged as Restricted. For the avoidance of doubt – participation in TERRE does not exclude an BM Participant from offering other services to The Company but on occasions if there are conflicts between services, The Company may have to flag these TERRE Bids as Restricted.

BC4.7			Instructing BM Participants

BC4.7.1		Communication from The Company to BM Participants

For the purposes of communication, an RR Instruction will follow the same format as a Bid-Offer Acceptance and so the rules of BC2.7 also apply for RR Instructions.

BC4.7.2		Creating RR Instructions from RR Acceptances

Results from the TERRE Central Platform are returned to The Company in the form of RR Acceptances.

RR Acceptances do not include physical ramps and so Run Up Rates and Run Down Rates will be used to create RR Instructions.

In order to comply with all of the RR Acceptances for a BM Participant, several RR Instructions may be required.

RR instructions will ramp BM Participants from their Commtted Level, hold them at the required output level, and then return the BM Participant back to the Commited Level.

The TERRE market wishes to incentivise RR Instructions which ramp within +/-5 minutes of the start and end of the TERRE Activation Periods. Hence, where possible,  Run Up Rates and Run Down Rates will be applied so that ramping is symmetric around the start and end of the TERRE Activiation Periods.

However the TERRE Product allows for up to 30 minute ramping to and from full activation and so for the first and final ramps up to 30 minutes of ramping can be used for creating an RR Instruction.

Details of how RR Instructions will be created can be found in the TERRE Instruction Guide.

BC4.7.3             Cases where RR Instructions may not be issued

In the time between receiving TERRE Bids and the RR Acceptances being returned to The Company, system conditions may require the issuing of a Bid Offer Acceptance to the BM Participant for which the RR Acceptance applies.

In these cases, it may be necessary to not issue an RR Instruction to the BM Participant or to modify the RR Instruction so that it is compatible with the Bid Offer Acceptance that has been previously been issued to the BM Participant.

This situation can only arise for a BM Participant which is also active in the Balancing Mechanism.

The following may apply:

- If the Bid Offer Acceptance is in the same direction as the RR Instruction but the MW levels of the RR Instruction are less than the Commited Level after the Bid Offer Acceptance is applied, the RR Instruction will not be issued.

- If the Bid Offer Acceptance is in the same direction as the RR Instruction but the MW levels of the RR Instruction are greater than the Commited Level after the Bid Offer Acceptance is applied the RR Instruction will be issued relative to the Commited Level.

- If the Bid Offer Acceptance is in the opposite direction to the RR Instruction the RR instruction will not be issued.

BC4.7.4             Infeasibility of RR Acceptances

If the RR Acceptances for an BM Participant are not consistent with the Physical Noifications and the Run Up Rates and Run Down Rates, then The Company will adjust the MW levels so that RR Instructions can be created using the declared parameters.

Details of how these infeasibility rules will be applied are contained in the TERRE Instruction Guide.

BC4.8               Publication of TERRE Data

BC4.8.1            Publication of Data at the European level

BC4.8.2            Publication of Data at the National level

The Company shall provide data in accordance with the requirements of the BSC. The following data items will be provided:

- TERRE Bids and details of those restricted

- Final Physical Notifications

- RR activations

- RR Instructions

- Interconnector Volumes per 15 minute period of the TERRE Activation Period

- The TERRE clearing price

- Volume of GB need met

BC4.9               Outages of computer systems leading to the suspension of the TERRE market

The TERRE market operates in short processing times meaning that Planned Maintenance Outages or unplanned computer system failures can result in the suspension of the TERRE market.

Suspension of the TERRE market in GB will occur in the following circumstances:

- Loss of communication from The Company to the TERRE Central Platform

- Failure of the TERRE Central Platform to produce RR Acceptances

- Loss of communication from the TERRE Central Platform to The Company

- Loss of electronic logging devices to a large number of BM Participants

< End of BC4 >