OPERATING CODE NO. 1

(OC1)

DEMAND FORECASTS

CONTENTS

(This contents page does not form part of the Grid Code)

Paragraph No/Title	Page Number

OC1.1   INTRODUCTION	2

OC1.2   OBJECTIVE	2

OC1.3   SCOPE	2

OC1.4   DATA REQUIRED BY THE COMPANY IN THE OPERATIONAL PLANNING PHASE	2

OC1.5   DATA REQUIRED BY THE COMPANY IN THE PROGRAMMING PHASE, CONTROL PHASE 	AND POST-CONTROL PHASE	3

OC1.6   THE COMPANY FORECASTS	5

OC1.1	INTRODUCTION

OC1.1.1	Operating Code No.1 ("OC1") is concerned with Demand forecasting for operational purposes. In order to match generation output with Demand for electricity it is necessary to undertake Demand forecasting. It is also necessary to undertake Demand forecasting of Reactive Power.

OC1.1.2	In the Operational Planning Phase, Demand forecasting shall be conducted by The Company taking account of Demand forecasts furnished by Network Operators, who shall provide The Company with information in the form set out in this OC1. The data supplied under the PC is also taken into account.

OC1.1.3	In the Programming Phase and Control Phase, The Company will conduct its own Demand forecasting taking into account information to be furnished by Suppliers and Network Operators and the other factors referred to in OC1.6.1.

OC1.1.4	In this OC1, the point of connection of the External Interconnection to the National Electricity Transmission System shall be considered as a Grid Supply Point. Reactive Power Demand includes the series Reactive losses of the User's System but excludes any network susceptance and any Reactive compensation on the User's System. The Company will obtain the lumped network susceptance and details of Reactive compensation from the requirements to submit data under the PC.

OC1.1.5	Data relating to Demand Control should include details relating to MW.

OC1.1.6	OC1 deals with the provision of data on Demand Control in the Operational Planning Phase, the Programming Phase and the Post-Control Phase, whereas OC6 (amongst other things) deals with the provision of data on Demand Control following the Programming Phase and in the Control Phase.

OC1.1.7	In this OC1, Year 0 means the current Financial Year at any time, Year 1 means the next Financial Year at any time, Year 2 means the Financial Year after Year 1, etc.

OC1.1.8	References in OC1 to data being supplied on a half hourly basis refer to it being supplied for each period of 30 minutes ending on the hour and half-hour in each hour.

OC1.2	OBJECTIVE

The objectives of OC1 are to:

OC1.2.1	enable the provision of data to The Company by Users in the Programming Phase, Control Phase and Post-Control Phase; and

OC1.2.2	provide for the factors to be taken into account by The Company when Demand forecasting in the Programming Phase and Control Phase.

OC1.3	SCOPE

OC1 applies to The Company and to Users which in this OC1 means:

(a)	Network Operators, and

(b)	Suppliers.

OC1.4	DATA REQUIRED BY THE COMPANY IN THE OPERATIONAL PLANNING PHASE

OC1.4.1	(a)	Each User, as specified in (b) below, shall provide The Company with the data requested in OC1.4.2 below.

(b)	The data will need to be supplied by each Network Operator directly connected to the National Electricity Transmission System in relation to Demand Control and in relation each Generator with respect to the output of Embedded Medium Power Stations within its System.

OC1.4.2	(a)	Data

By calendar week 28 each year each Network Operator will provide to The Company in writing the forecast information listed in (c) below for the current Financial Year and each of the succeeding five Financial Years.

(b)	Data Providers

In circumstances when the busbar arrangement at a Grid Supply Point is expected to be operated in separate sections, separate sets of forecast information for each section will be provided to The Company.

(c)	Embedded Medium Power Station Output and Demand Control

For the specified time of the annual peak half hour National Electricity Transmission System Demand, as specified by The Company under PC.A.5.2.2, the output of Embedded Medium Power Stations and forecasts of Demand to be relieved by Demand Control on a Grid Supply Point basis giving details of the amount and duration of the Demand Control.

OC1.5	DATA REQUIRED BY THE COMPANY IN THE PROGRAMMING PHASE, CONTROL PHASE AND POST-CONTROL PHASE

OC1.5.1	Programming Phase

For the period of 2 to 8 weeks ahead the following will be supplied to The Company in writing by 1000 hours each Monday:

(a)	Demand Control

Each Network Operator will supply MW profiles of the amount and duration of their proposed use of Demand Control which may result in a Demand change equal to or greater than the Demand Control Notification Level (averaged over any half hour on any Grid Supply Point) on a half hourly and Grid Supply Point basis;

(b)	Medium Power Station Operation

Each Network Operator will, if reasonably required by The Company, supply MW schedules for the operation of Embedded Medium Power Stations within its System on a half hourly and Grid Supply Point basis.

OC1.5.2	For the period 2 to 12 days ahead the following will be supplied to The Company in writing by 1200 hours each Wednesday:

(a)	Demand Control

Each Network Operator will supply MW profiles of the amount and duration of their proposed use of Demand Control which may result in a Demand change equal to or greater than the Demand Control Notification Level (averaged over any half hour on any Grid Supply Point) on a half hourly and Grid Supply Point basis;

(b)	Medium Power Station Operation

Each Network Operator will, if reasonably required by The Company, supply MW schedules for the operation of Embedded Medium Power Stations within its System on a half hourly and Grid Supply Point basis.

OC1.5.3	Medium Power Station Output

Each Network Operator will, if reasonably required by The Company, supply The Company with MW schedules for the operation of Embedded Medium Power Stations within its System on a half hourly and Grid Supply Point basis in writing by 1000 hours each day (or such other time specified by The Company from time to time) for the next day (except that it will be for the next 3 days on Fridays and 2 days on Saturdays and may be longer (as specified by The Company at least one week in advance) to cover holiday periods);

OC1.5.4	Other Codes

Under OC6 each Network Operator will notify The Company of their proposed use of Demand Control (which may result in a Demand change equal to or greater than the Demand Control Notification Level), and under BC1, each Supplier will notify The Company of their proposed use of Customer Demand Management (which may result in a Demand change equal to or greater than the Customer Demand Management Notification Level) in this timescale.

OC1.5.5	Control Phase

OC1.5.5.1	Demand Control

Under OC6, each Network Operator will notify The Company of any Demand Control proposed by itself which may result in a Demand change equal to or greater than the Demand Control Notification Level averaged over any half hour on any Grid Supply Point which is planned after 1000 hours, and of any changes to the planned Demand Control notified to The Company prior to 1000 hours as soon as possible after the formulation of the new plans;

OC1.5.5.2	Customer Demand Management

(a)	Each Supplier will notify The Company of any Customer Demand Management proposed by itself which may result in a Demand change equal to or greater than the Customer Demand Management Notification Level averaged over any half hour on any Grid Supply Point which is planned to occur at any time in the Control Phase and of any changes to the planned Customer Demand Management already notified to The Company as soon as possible after the formulation of the new plans.

(b)	The following information is required on a Grid Supply Point and half-hourly basis:

(i)	the proposed date, time and duration of implementation of Customer Demand Management; and

(ii)	the proposed reduction in Demand by use of Customer Demand Management.

OC1.5.5.3	Load Management Blocks

In Scotland, by 11:00 each day, each Supplier who controls a Load Management Block of Demand with a capacity of 5MW or more shall submit to The Company a schedule of its proposed switching times and profiles in respect of each block for the next day.

OC1.5.6	Post-Control Phase

The following will be supplied to The Company in writing by 0600 hours each day in respect of Active Power data and by 1000 hours each day in respect of Reactive Power data:

(a)	Demand Control

Each Network Operator will supply MW profiles for the previous calendar day of the amount and duration of Demand reduction achieved by itself from the use of Demand Control equal to or greater than the Demand Control Notification Level (averaged over any half hour on any Grid Supply Point), on a half hourly and Grid Supply Point basis.

(b)	Customer Demand Management

Each Supplier will supply MW profiles of the amount and duration of Demand reduction achieved by itself from the use of Customer Demand Management equal to or greater than the Customer Demand Management Notification Level (averaged over any half hour on any Grid Supply Point) on a half hourly and Grid Supply Point basis during the previous calendar day.

OC1.6	THE COMPANY FORECASTS

OC1.6.1	The following factors will be taken into account by The Company when conducting National Electricity Transmission System Demand forecasting in the Programming Phase and Control Phase:

(a)	Historic Demand data (this includes National Electricity Transmission System Losses).

(b)	Weather forecasts and the current and historic weather conditions.

(c)	The incidence of major events or activities which are known to The Company in advance.

(d)	Anticipated interconnection flows across External Interconnections.

(e)	Demand Control equal to or greater than the Demand Control Notification Level (averaged over any half hour at any Grid Supply Point) proposed to be exercised by Network Operators and of which The Company has been informed.

(f)	Customer Demand Management equal to or greater than the Customer Demand Management Notification Level (averaged over any half hour at any Grid Supply point) proposed to be exercised by Suppliers and of which The Company has been informed.

(g)	Other information supplied by Users.

(h)	Anticipated Pumped Storage Unit demand.

(i)	the sensitivity of Demand to anticipated market prices for electricity.

(j)	BM Unit Data submitted by BM Participants to The Company in accordance with the provisions of BC1 and BC2.

(k)	Demand taken by Station Transformers

(l) 	Anticipated Electricity Storage Module demand

OC1.6.2	Taking into account the factors specified in OC1.6.1 The Company uses Demand forecast methodology to produce forecasts of National Electricity Transmission System Demand. A written record of the use of the methodology must be kept by The Company for a period of at least 12 months.

OC1.6.3	The methodology will be based upon factors (a), (b) and (c) above to produce, by statistical means, unbiased forecasts of National Demand. National Electricity Transmission System Demand will be calculated from these forecasts but will also take into account factors (d), (e), (f), (g), (h), (i) and (j) above. No other factors are taken into account by The Company, and it will base its National Electricity Transmission System Demand forecasts on those factors only.

< END OF OPERATING CODE NO. 1 >