BALANCING CODE NO. 6 (BC6)

MARI PROCESSES

CONTENTS

(This contents page does not form part of the Grid Code)

Paragraph No/Title	Page Number

BC6.1	INTRODUCTION	2

BC6.2	OBJECTIVE	2

BC6.3	SCOPE	2

BC6.4	REQUIREMENTS FOR BM PARTICIPANTS WHO WISH TO PARTICIPATE IN MARI	2

BC6.4.1	All BM Participants who wish to participate in MARI must have successfully completed the prequalification process to be a mFRR provider as detailed in BC6.	2

BC6.4.2	All BM Participants who wish to participate in  MARI must have the following capabilities;-	2

BC6.4.3	Prequalification Timelines	3

BC6.4.4	Requalification criteria	3

BC6.5	SUBMISSION OF MARI RELATED DATA BY BM PARTICIPANTS	3

BC6.5.1	Communication from BM Participants to The Company	3

BC6.5.2	mFRR Provider Data submissions before  MARI Gate Closure	4

BC6.5.3	Re-submission of parameters by BM Participants before  MARI Gate Closure	5

BC6.5.4	Defaulting rules for  MARI Bids	5

BC6.6.1	Cases where a MARI Bid will be Restricted	5

BC6.7	 Instructing BM Participants	5

BC6.7.1	Communication from The Company to BM Participants	5

BC6.7.2	Creating  mFRR Instructions from mFRR Acceptances	5

BC6.7.3	 Cases where mFRR Instructions may not be issued	6

BC6.7.4	Infeasibility of mFRR Acceptances	6

BC6.8   Publication of MARI Data	6

BC6.8.1      Publication of Data at the European level	6

BC6.8.2      Publication of Data at the National level	6

BC6.9	Outages of computer systems leading to the suspension of the MARI market	7

BC6.10 	MARI Market Suspension	7

# BC6.1	INTRODUCTION

Balancing Code No 6 (BC6) sets out the procedures for:

(a) 	prequalifation requirements for participation in MARI by BM Participants;

(b) 	submission of data by BM Participants wishing to take part in MARI;

(c) 	validation of data from BM Participants wishing to take part in MARI;

(d) 	issuing of  mFRR Instructions;

(e) 	publication of  MARI related data.

# BC6.2	OBJECTIVE

This procedure facilitates the participation of BM Participants in the MARI market. Participation in  MARI is voluntary for BM Participants.

# BC6.3	SCOPE

BC6 applies to :-

- The Company;

- BM Participants;

(b)	Externally Interconnected System Operators; and

(c)	Network Operators.

# BC6.4	REQUIREMENTS FOR BM PARTICIPANTS WHO WISH TO PARTICIPATE IN MARI

The Company shall ensure that each relevant Balancing Service prequalification process shall, as a minimum, require the  mFRR provider to submit a self-certification of the mFRR minimum technical requirements as defined in. BC6.4.1 and BC6.4.2.

### BC6.4.1	All BM Participants who wish to participate in MARI must have successfully completed the prequalification process to be a mFRR provider as detailed in BC6.

### BC6.4.2	All BM Participants who wish to participate in  MARI must have the following capabilities;-

- BM Participants must have the ability to submit data and receive instructions by the use of electronic data communication facilities as provided for in CC.6.5.8 or ECC.6.5.8. In addition, BM Participants who wish to participate in MARI are also required to install Control Telephony (irrespective of whether they form part of a Large Power Station) in accordance with the requirements of CC.6.5.2 – CC.6.5.5 or ECC.6.5.2 - ECC.6.5.5

- BM Participants must be capable of following an mFRR Instruction issued by The Company

- BM Participants must be able to provide Physical Notifications

- BM Participants must be able to provide a subset of Dynamic Parameters (as detailed in  BC6.5.2)

- BM Participants must provide operational metering for their total output and for any individual component that may have an output greater than 1MW. This metering must have the following accuracy;

  - For a BM Unit with either Generation Capacity greater than 100MW or Demand Capacity greater than 100MW, a metering accuracy better than 0.5%

  - For a BM Unit with a Generation Capacity greater than 10MW but less than or equal to 100MW or a Demand Capacity greater than 10MW but less than or equal to 100MW, a metering accuracy better than 1%

  - For all other BM Units, an accuracy better than 2.5% is required

  - The requirements of the operational metering scheme shall be specified in the Bi-lateral Agreement

- BM Participants must have the ability to inform The Company if their availability changes using Export and Import Limits

- For BM Participants connected within a User System, BM Participants must be capable of informing Network Operators of their availability and activation in realtime if required

### BC6.4.3	Prequalification Timelines

European Regulation 2017/1485 gives the following minimum timescales for the prequalification process.

(a)	Within 8 weeks of a formal application from the BM Participant, The Company shall confirm the application is complete (from the perspective of information provision).

(b)	If the application is incomplete, the BM Participant shall provide the missing evidence within 4 weeks of the request from The Company or it will be presumed that the application has been withdrawn.

(c)	Within 3 months of confirming that all information has been provided, The Company shall confirm if the potential BM Participant meets the requirements in BC6.4.2. For the avoidance of doubt – The Company will not carry out independent tests but will review the evidence provided.

### BC6.4.4	Requalification criteria

Under certain conditions, an BM Participant must requalify

(a)	Every five years, a BM Participant must requalify to the technical requirements in BC6.4.2 and according to the timescales in BC6.4.3.

(b)	If at any time, a BM Participant becomes aware of changes to the configuration forming the BM Unit that means the minimum technical requirements in BC6.4.2 can no longer be met, that BM Participant must withdraw from MARI and must requalify.

# BC6.5	SUBMISSION OF MARI RELATED DATA BY BM PARTICIPANTS

### BC6.5.1	Communication from BM Participants to The Company

(a)	Submission of data specified in BC6.5.2 will be by use of electronic data communications facilities, as provided for in CC.6.5.8 or ECC.6.5.8.

(b)	In the event of a failure of the electronic data communication facilities, the data used in the MARI auction will be based on the most recent data received and acknowledged by The Company. In the event of missing data, it will be assumed the BM Participant did not wish to submit data for the relevant MARI Auction  Period.

(c)	Planned Maintenance Outages will normally be arranged to take place during periods of low data transfer activity.

(d)	Upon any Planned Maintenance Outage, or following an unplanned outage described in BC6.5.1 (b) (where it is termed a "failure") in relation to a pre- MARI  Gate Closure:

(i)	If a BM Participant has submitted Physical Notifications and a MARI Bid for a  MARI Auction Period, the BM Participant should continue to act in relation to any period of time in accordance with the Physical Notifications current at the time of the start of the Planned Maintenance Outage or the computer system failure in relation to each such period of time subject to the provisions of BC2.5.1. Depending on when in relation to MARI Gate Closure, the planned or unplanned maintenance outage arises, MARI Bids  will either be in preparation for the relevant output in real time, or will be in operation in real time. No further submissions of BM Participants data should be attempted. Plant failure or similar problems causing significant deviation from Physical Notification should be notified to The Company by the submission of a revision to Export and Import Limits in relation to the  mFRR Provider so affected;

(ii) 	No data will be transferred from The Company to the Balancing Mechanism Reporting Agent (BMRA) until the communication facilities are re-established.

### BC6.5.2	mFRR Provider Data submissions before  MARI Gate Closure

To participate in a MARI auction, a BM Participant must have prequalified and must submit a  MARI Bid covering at least one of the  MARI Activation Periods within the  MARI Auction Period.

In addition to a valid  MARI Bid, a sub-set of Balancing Mechanism parameters are also required covering the  MARI Auction Period and the Settlement Periods immediately before and after the  MARI Auction Period (to allow ramping before and after).

If a BM Participant is active in the Balancing Mechanism, the only additional data needed to participate in a  MARI  auction is a valid MARI Bid covering the relevant times.

For a BM Participant that is not active in the Balancing Mechanism, the following subset of parameters are required with exceptions as noted below:

(a)	Physical Notifications

Physical Notifications follow the same format and rules as covered in BC1 and BC2 with the following exceptions;

- A BM Participant that is not active in the Balancing Mechanism but wishes to participate in MARI is only required to have submitted Physical Notifications covering the  MARI Auction Period and the Settlement Periods immediately before and after the  MARI Auction Period for which they have submitted a MARI Bid.

- Defaulting rules as described in the Data Validation, Consistency and Defaulting Rules will only apply to Settlement Periods for which the BM Participant previously submitted Physical Notifications for the previous Operational Day.

(b)	Export and Import Limits

For a BM Participant that is not active in the Balancing Mechanism but wishes to participate in  MARI, these are the same as described in BC1 and BC2

- Run Up Rate and Run Down Rates

For a BM Participant that is not active in the Balancing Mechanism but wishes to participate in  MARI these are the same as described in BC1 and BC2

- For a BM Participant that is not active in the Balancing Mechanism but wishes to participate in  MARI the other Dynamic Parameters listed in BC1.A.1.5 are not required

MARI Bids must follow the formats and rules in the MARI Data Validation and Consistency Rules.

### BC6.5.3	Re-submission of parameters by BM Participants before  MARI Gate Closure

The rules outlined in BC1 and BC2 for the revision of Physical Notifications, Export and Import Limits, Run Up Rates and Run Down Rates also apply for MARI.

MARI Bids can be revised up to  MARI Gate Closure in order to be used in the MARI auction (as described in the MARI Data Validation and Consistency Rules).

### BC6.5.4	Defaulting rules for  MARI Bids

MARI Bids will not be defaulted using previously submitted values. This is due to the ability to link MARI Bids and the re-use of sequence numbers. Hence a BM Participant wishing to participate in a particular  MARI auction must submit  mFRR Bids specifically covering the relevant  MARI Activation Periods.

BC6.6 	Processing of MARI  Bids before passing to the MARI Central Platform

### BC6.6.1	Cases where a MARI Bid will be Restricted

MARI Bids will be passed to the MARI Central Platform but will be flagged as Restricted 	under the following cases;

- Data within the submission does not conform to formats required as detailed in the  MARI Data Validation and Consistency Rules (e.g. missing or incorrect keywords, data in the wrong order, corrupted files etc.)

- If a MARI Bid does not have a corresponding Physical Notification the  MARI Bid 	will be flagged as Restricted

- If a MARI Bid will result in violating a System Constraint it will be flagged as 	Restricted

- If a BM Participant has already been instructed for an Ancillary Service or for 	Reserve, a  MARI Bid may need to be flagged as Restricted. For the avoidance of 	doubt – participation in  MARI does not exclude an BM Participant from offering 	other services to The Company but on occasions if there are conflicts between 	services, The Company may have to flag these  MARI Bids as Restricted.

# BC6.7		Instructing BM Participants

### BC6.7.1	Communication from The Company to BM Participants

For the purposes of communication an mFRR Instruction will follow the same format as a Bid-Offer Acceptance and so the rules of BC2.7 also apply for mFRR Instructions.

### BC6.7.2	Creating  mFRR Instructions from mFRR Acceptances

Results from the  MARI Central Platform are returned to The Company in the form of mFRR Acceptances.

mFRR Acceptances do not include physical ramps and so Run Up Rates and Run Down Rates will be used to create mFRR Instructions.

In order to comply with all of the mFRR Acceptances for a BM Participant several mFRR Instructions may be required.

mFRR instructions will ramp BM Participants from their Committed Level, hold them at the required output level, and then return the BM Participant back to the Committed Level.

The MARI market wishes to incentivise mFRR Instructions which ramp within +/-5 minutes of the start and end of the MARI Activation Periods.  Hence, where possible,  Run Up Rates and Run Down Rates will be applied so that ramping is symmetric around the start and end of the  MARI Activation Periods.

However the MARI Product allows for up to 12.5 minute ramping to and from full activation and so for the first and final ramps up to 12.5 minutes of ramping can be used for creating an mFRR Instruction.

Details of how mFRR Instructions will be created can be found in the NGESO MARI Reference .

### BC6.7.3	 Cases where mFRR Instructions may not be issued

In the time between receiving MARI Bids and the mFRR Acceptances being returned to The Company, system conditions may require the issuing of a Bid Offer Acceptance to the BM Participant for which the mFRR Acceptance applies.

In these cases it may be necessary to not issue an mFRR Instruction to the BM Participant or to modify the mFRR Instruction so that it is compatible with the Bid Offer Acceptance that has been previously been issued to the BM Participant.

This situation can only arise for a BM Participant which is also active in the Balancing Mechanism.

The following may apply:

- If the Bid Offer Acceptance is in the same direction as the  mFRR Instruction but the MW levels of the mFRR Instruction are less than the Committed Level after the Bid Offer Acceptance is applied, the  mFRR Instruction will not be issued.

- If the Bid Offer Acceptance is in the same direction as the mFRR Instruction but the MW levels of the  mFRR Instruction are greater than the Committed Level after the Bid Offer Acceptance is applied, the mFRR Instruction will be issued relative to the Committed Level.

- If the Bid Offer Acceptance is in the opposite direction to the mFRR Instruction the mFRR instruction will not be issued.

### BC6.7.4	Infeasibility of mFRR Acceptances

If the mFRR Acceptances for an BM Participant are not consistent with the Physical Notifications and the Run Up Rates and Run Down Rates, then The Company will adjust the MW levels so that mFRR Instructions can be created using the declared parameters.

Details of how these infeasibility rules will be applied are contained in the NGESO MARI Reference Document.

# BC6.8   Publication of MARI Data

### BC6.8.1      Publication of Data at the European level

### BC6.8.2      Publication of Data at the National level

The Company shall provide data in accordance with the requirements of the BSC. The following data items will be provided:

- MARI Bids and details of those restricted

- Final Physical Notifications

- mFRR Activations

- mFRR Instructions

- Interconnector Volumes per 12.5 minute period of the MARI Activation Period

- The MARI clearing price

- Volume of GB need met

- MARI Available Transfer Capacity per 12.5 minute period of the MARI Activation Period

- MARI  Desired Flow Range per 12.5 minute period of the MARI Activation Period

# BC6.9	Outages of computer systems leading to the suspension of the MARI market

The MARI market operates in short processing times meaning that Planned Maintenance Outages or unplanned computer system failures can result in the suspension of the MARI market.

Suspension of the  MARI market in GB will occur in the following circumstances:

- Loss of communication from The Company to the  MARI Central Platform

- Failure of the  MARI Central Platform to produce mFRR Acceptances

- (c) Loss of communication from the  MARI Central Platform to The CompanyLoss of electronic logging devices to a large number of BM Participants

# BC6.10 	MARI Market Suspension

The application of the MARI market shall be suspended in GB when one of the following circumstances arises:

- Suspension of the Balancing Mechanism in accordance with OC9.4.6; or

- Outages of computer systems leading to the suspension of the MARI market as provided for in BC6.9; or

- Operators of the MARI Central Platform notify The Company that the MARI market has been or is to be suspended.

Where the MARI market has been suspended as a result of items (a), (b) or (c) above, The Company will as soon as reasonably practical, inform Users and the BSCCo that the MARI market is to be or has been suspended.  The Company will notify Users and the BSSCo if the MARI market suspension arose as a result of a Black Start event or another condition in accordance with the requirements of the BSC.

The Company shall (as soon as is practicable) determine, in its reasonable opinion, the time and date from when the MARI market is to be suspended.  The Company shall also notify Users and the BSCCo of the time of MARI market suspension and the reason for the suspension.

Where the MARI market has been suspended, it will not be resumed until the start of a defined Settlement Period which shall be determined:-

- by the BSC Panel in accordance with section G3.1.8 of the BSC (in the case of a Black Start event); or

- by section Q.5.A of the BSC (in the case of MARI Market suspension for any other reason other than Black Start).

In the case of MARI Market suspension as a result of a Black Start event, as provided for under BC6.10(a), Users shall use reasonable endeavours to submit MARI Bids ten hours prior to the start of the Settlement Period determined by the BSC Panel in accordance with paragraph G3.1.8 of the BSC and as notified by The Company to Users in preparation for return to resumption of the MARI market.

In the case of MARI Market suspension as a result of another event as provided for under BC6.10(b) or BC6.10(c), Users shall use reasonable endeavours to submit MARI Bids as soon as possible after notification from The Company of the Settlement Period from when the MARI market is to be re-established.