Historic Demand Data

Description of contents

This document explains what is included in the historic demand data files (DemandData_YYYY.csv and DemandData_Update.csv) as published on the National grid website:

https://www.nationalgrid.com/uk/electricity/market-operations-and-data/data-explorer

| Data Item | Description |
| --- | --- |
| SETTLEMENT_DATE | Settlement Date |
| SETTLEMENT_PERIOD | Settlement Period |
| ND | National Demand This is the Great Britain generation requirement and is equivalent to the Initial National Demand Outturn (INDO) and National Demand Forecast as published on BM Reports. National Demand is the sum of metered generation, but excludes generation required to meet station load, pump storage pumping and interconnector exports. National Demand is calculated as a sum of generation based on National Grid operational generation metering. |
| FORECAST_ACTUAL_INDICATOR | Indication of whether data is out-turn (A) or forecast (F). |
| I014_ND | Equivalent to ND (above) but calculated using settlement metered generation data from the I014 file where available. |
| TSD | Transmission System Demand This is the Transmission System generation requirement and is equivalent to the Initial Transmission System Outturn (ITSDO) and Transmission System Demand Forecast on BM Reports. Transmission System Demand is equal to the ND plus the additional generation required to meet station load, pump storage pumping and interconnector exports. Transmission System Demand is calculated using National Grid operational metering. Note that the Transmission System Demand includes an estimate of station load of 500MW in BST and 600MW in GMT. |
| I014_TSD | Equivalent to TSD (above), but calculated using settlement metered generation data from the I014 file where available. |
| ENGLAND_WALES_DEMAND | England and Wales Demand, as ND above but on an England and Wales basis. |
| EMBEDDED_WIND_GENERATION | Estimated Embedded Wind Generation This is an estimate of the GB wind generation from wind farms which do not have Transmission System metering installed. These wind farms are embedded in the distribution network and invisible to National Grid. Their effect is to suppress the electricity demand during periods of high wind. The true output of these generators is not known so an estimate is provided based on National Grid’s best model. Note that embedded wind farms which do have Transmission System metering are not included in this total. For future dates a forecast value is shown. This is equivalent to the data that feeds into the National Demand forecast published on BM Reports. |
| EMBEDDED_WIND_CAPACITY | Estimated Embedded Wind Capacity This is National Grid’s best view of the installed embedded wind capacity in GB. This is based on publically available information compiled from a variety of sources and is not the definitive view. It is consistent with the generation estimate provided above. |
| EMBEDDED_SOLAR_GENERATION | Estimated Embedded Solar Generation As embedded wind generation above, but for solar generation. |
| EMBEDDED_SOLAR_CAPACITY | Embedded Solar Capacity As embedded wind capacity above, but for solar generation. |
| NON_BM_STOR | Non-BM Short-Term Operating Reserve Operating reserve for units that are not included in the ND generator definition. This can be in the form of generation or demand reduction. |
| PUMP_STORAGE_PUMPING | Pump Storage Pumping The demand due to pumping at hydro pump storage units; the -ve signifies pumping load. |
| I014_PUMP_STORAGE_PUMPING | As above, but calculated based on settlement data from the I014 file where available. |
| FRENCH_IMPORT BRITNED_IMPORT MOYLE_IMPORT EAST_WEST_IMPORT | Interconnector Flow The flow on the respective interconnector. -ve signifies export power out from GB; +ve signifies import power into GB. |
| I014_FRENCH_IMPORT I014_BRITNED_IMPORT I014_MOYLE_IMPORT I014_EAST_WEST_IMPORT | Each as above, but calculated based on settlement data from the I014 file where available. |

If you have any questions or feedback please contact:

Email: .box.demand.forecast.queries